SUPPLEMENTARY INFORMATION

Sustainable Pollutant Removal through Integrated Biological, Physical, Electrochemical Treatment: Resource Recovery and LCA Study

Keywords: Vinasse wastewater; anaerobic digestion; aerobic oxidation; electrochemical oxidation; life cycle assessment; resource recovery

# A.1. Theoretical Estimation of CO2 and H2O Generated in Aerobic Process

## 1. Aerobic Treatment Data

| Parameter | Value |
| --- | --- |
| Initial BOD concentration | 344 mg/L |
| Final BOD concentration | 102 mg/L |
| BOD removed | 242 mg/L |
| Reactor volume | 24.24 L wastewater + 60 L activated sludge |

## 2. Aerobic Oxidation Reaction

During aerobic biological treatment, microorganisms oxidize biodegradable organic matter using dissolved oxygen as the electron acceptor. The simplified aerobic reaction can be expressed as:

Organic Matter + O2 → CO2 + H2O + bio-substance

## 3. Estimation of CO2 Generation

Theoretical estimation assumes that 1 g of organic matter oxidized can generate approximately 1.375 g CO2 after complete mineralization (Mr CO2/Mr O2).

BOD removed = 344 − 102 = 242 mg/L = 0.242 g/L

Estimated CO2 generated per liter:

CO2 = 0.242 g/L × 1.375 g = 0.333 g CO2/L

For 24.24 L wastewater:

Total CO2 = 0.333 g/L × 24.24 L = 8.08 g CO2

## 4. Estimation of H2O Generation

Assuming that water generation follows approximately the same molar formation trend as CO2 during aerobic mineralization:

Moles of CO2 = 8.08 g / 44 g/mol = 0.1836 mol

Estimated H2O generated:

H2O = 0.1836 mol × 18 g/mol = 3.3048 g H2O

# A.2. Theoretical Estimation of CO2 and H2O Generation in Electrochemical Process

## 1. Basis of Calculation

| Parameter | Value |
| --- | --- |
| Initial COD before electrochemical treatment | 676.304 mg/L |
| Final COD after electrochemical treatment | 307.325 mg/L |
| Volume of vinasse treated | 16.16 L |
| COD removed | 368.979 mg/L |

## 2. Aerobic Oxidation Reaction

During aerobic biological treatment, biodegradable organic compounds are oxidized by aerobic microorganisms using oxygen as the electron acceptor. The simplified reaction can be represented as follows:

Organic Matter + O2 → CO2 + H2O

This reaction produces carbon dioxide and water as the main end-products of organic matter mineralization.

## 3. Electrochemical Oxidation Reaction

In electrochemical oxidation, hydroxyl radicals (•OH) and reactive chlorine species generated on the MMO electrode surface oxidize dissolved organic compounds into CO2 and H2O.

Organic Matter + •OH → CO2 + H2O + Intermediates

## 4. CO2 Generation Calculation

Theoretical estimation assumes that 1 g COD removed produces approximately 1.375 g CO2.

COD removed = 676.304 − 307.325 = 368.979 mg/L = 0.368979 g/L

Theoretical CO2 generation per liter:

CO2 = 0.368979 × 1.375 = 0.507 g CO2/L

For 16.16 L vinasse:

Total CO2 = 0.507 g/L × 16.16 L = 8.193 g CO2

## 5. H2O Generation Calculation

Assuming that water formation approximately follows the molar formation of CO2 during organic mineralization:

Moles of CO2 = 8.193 g / 44 g/mol = 0.186 mol

Assuming equal molar H2O production:

Mass of H2O = 0.186 mol × 18 g/mol = 3.348 g H2O

# A.3. Theoretical Estimation of Cl2 Generated in Electrochemical Vinasse Treatment

## Reaction

The calculation is based on Faraday’s law and the chlorine evolution reaction at the anode:

2Cl⁻ → Cl2 + 2e⁻

The reactor conditions used in the study were an applied voltage of 8 V, current of 8–10 A, reaction time of 60 min, and a Ti/RuO2 anode paired with a stainless-steel cathode in NaCl electrolyte.

## Assumptions and basis of calculation

| Parameter | Value |
| --- | --- |
| Reaction time | 60 min = 3600 s |
| Electrons transferred | n = 2 |
| Faraday constant | F = 96485 C/mol e⁻ |
| Molar mass of Cl2 | M = 70.906 g/mol |
| Current range | 8–10 A |

The theoretical chlorine generation was calculated using:

m = (I × t × M) / (n × F)

where m is the mass of Cl2 generated, I is current, t is time, M is molar mass, n is number of electrons, and F is Faraday constant.

## Calculation

For 8 A:

m = (8 × 3600 × 70.906) / (2 × 96485) = 10.58 g Cl2

For 10 A:

m = (10 × 3600 × 70.906) / (2 × 96485) = 13.23 g Cl2

Therefore, the theoretical amount of Cl2 generated during 60 min of electrolysis is approximately 10.58–13.23 g, assuming 100% current efficiency and that all anodic charge is consumed by chlorine evolution.

For a 2 L reactor, this is equivalent to about 5.29–6.61 g Cl2 per liter of treated solution. In practice, the measured amount can be lower because part of the generated chlorine dissolves in water and converts into HClO and ClO⁻:

Cl2 + H2O ⇌ HClO + Cl⁻ + H⁺

HClO ⇌ ClO⁻ + H⁺

Thus, the calculated value should be treated as the theoretical maximum chlorine production at the anode, not necessarily the free gaseous Cl2 released to air.

# A.4 Stoichiometric Biogas Burner Combustion Heat Calculation

This report presents the stoichiometric combustion analysis and heat-generation calculation for a burner system that utilizes CH4 and CO generated by the vinasse treatment process. The calculation includes the stoichiometric air requirement, a 20% excess-air condition, the theoretical combustion heat, and the actual useful heat, considering burner efficiency.

## Gas Composition Data

| Gas | Volume (m3) | Mass (kg) | Moles (mol) |
| --- | --- | --- | --- |
| CH4 | 0.00379904 | 0.00249217 | 0.1554 |
| CO2 | 0.00078809 | 0.00145009 | - |
| CO | 7.3653E-05 | 8.4333E-05 | 0.00301 |

## Stoichiometric Combustion Reactions

CH4 + 2O2 → CO2 + 2H2O

CO + ½O2 → CO2

## Stoichiometric Oxygen Requirement

n(CH4) = 0.00249217 / 0.01604 = 0.1554 mol

n(CO) = 8.4333×10-5 / 0.02801 = 0.00301 mol

n(O2) = 2(0.1554) + 0.5(0.00301) = 0.312 mol O2

n(CO2) = n(CH4) + n(CO) = 0.1584 mol

m(CO2) = 0.1584 × 44 = 6.97 g CO2

m(CO2)total = 6.97 + 1.45 = 8.42 g = 0.00842 kg

## Air Requirement with 20% Excess Air

n(air) = 0.312 / 0.21 = 1.487 mol air

n(air,20%EA) = 1.487 × 1.2 = 1.784 mol air

m(air) = 1.784 × 28.97 = 51.69 g = 0.0517 kg

## Stoichiometric Combustion Heat

| Gas | Mass (kg) | LHV (MJ/kg) | Heat Generated (MJ) |
| --- | --- | --- | --- |
| CH4 | 0.00249217 | 50 | 0.1246 |
| CO | 8.4333E-05 | 10.1 | 0.000852 |

Q(total) = 0.1246 + 0.000852 = 0.1255 MJ = 125.5 kJ

## Burner Efficiency

Burner efficiency = 45%

Q(useful) = 0.1255 × 0.45 = 0.0565 MJ = 56.5 kJ

## Summary of Results

| Parameter | Value |
| --- | --- |
| Stoichiometric oxygen required | 0.312 mol O2 |
| Stoichiometric air required | 1.487 mol air |
| Air supplied with 20% excess air | 1.784 mol air |
| Mass of air supplied | 0.0517 kg |
| Mass of final CO2 | 0.00842 kg |
| Theoretical combustion heat | 125.5 kJ |
| Useful burner heat (45% efficiency) | 56.5 kJ |

Table S1 Chemical analysis results of solid organic fertilizer

| No | Parameter | Test Results | Technical Requirements* |
| --- | --- | --- | --- |
| 1 | pH | 8.29 | 4 - 9 |
| 2 | NTotal | 1.681% | Min. 2% (accumulation) |
| 3 | P2O5 | 0.145% |  |
| 4 | K2O | 2.145% |  |
| 5 | Moisture | 8.71% | 8 - 20% |
| 6 | Organic Carbon | 31.21% | Min. 15% |
| 7 | C/N ratio | 18.57 | < 25 |

*Technical Requirements based on the Decree of the Minister of Agriculture of the Republic of Indonesia Number 261/KPTS/SR.310/M/4/2019 concerning Minimum Technical Requirements for Organic Fertilizers, Biofertilizers, and Soil Improvers.

Table S2 LCI of sludge and biogas treatment recovery processes

| Exchanges | Scenario 1 | Scenario 2 | Scenario 3 | Scenario 4 | Unit | Stream data | Database | Data Quality |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Materials |  |  |  |  |  |  |  |  |
| Sludge | - | -8.08 | - | -8.08 | kg | Reference flow |  |  |
| Cattle manure | - | -8.08 | - | -8.08 | kg | Manure, solid, cattle {GLO}¦ market for manure, solid, cattle ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Rice bran | - | -8.08 | - | -8.08 | kg | Rice bran (raw), at processing {CN} Mass, S | Agri-foodprint | 2,1,1,3,na |
| Azolla microphylla | - | -0.008 | - | -0.008 | m3 | Green manure, organic, until April {RoW}¦ green manure growing, organic, until April ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Sawdust | - | -24.24 | - | -24.24 | kg | Sawdust, loose, wet, measured as dry mass {RoW}¦ sawing, hardwood ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Anaerobic air emission |  |  |  |  |  | Reference flow |  |  |
| CH4 | - | - | 2.49 | 2.49 | g |  | SimaPro flow | 2,1,1,1,na |
| CO2 | - | - | 1.45 | 1.45 | g |  | SimaPro flow | 2,1,1,1,na |
| CO | - | - | 0.05 | 0.05 | g |  | SimaPro flow | 2,1,1,1,na |
| Substitutes |  |  |  |  |  |  |  |  |
| Urea, as N* | - | 0.951 | - | 0.951 | kg | Inorganic nitrogen fertiliser, as N {ID}¦ market for inorganic nitrogen fertiliser, as N ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Rock phosphate, as P2O5* | - | 0.082 | - | 0.082 | kg | Inorganic phosphorus fertiliser, as P2O5 {ID}¦ market for inorganic phosphorus fertiliser, as P2O5 ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Potassium chloride, as K2O* | - | 1.213 | - | 1.213 | kg | Inorganic potassium fertiliser, as K2O {ID}¦ market for inorganic potassium fertiliser, as K2O ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Heat (45% burner eff) | - | - | 56.5 | 56.5 | kJ | Heat, central or small-scale, other than natural gas {ID}¦ heat and power co-generation, biogas, gas engine ¦ Cut-off, S |  |  |
| Emissions |  |  |  |  |  |  |  |  |
| CO2 | - | - | 8.42 | 8.42 | g |  | SimaPro flow | 2,1,1,1,na |
| H2O | - | - | 5.60 | 5.60 | g |  | SimaPro flow | 2,1,1,1,na |

*The fertilizer substitution was applied based on its chemical analysis, as mentioned in Table S1

Table S3 Ecoinvent data quality

| Score | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- |
| 1 Reliability | Verified data based on measurements | Verified data, partly based on assumptions OR non-verified data based on measurements | Non-verified data partly based on qualified estimates | Qualified estimate (e.g., by an industrial expert); data derived from theoretical information (stoichiometry, enthalpy, etc.) | Non-qualified estimate |
|  | 1.00 | 1.05 | 1.10 | 1.20 | 1.50 |
| 2 Completeness | Representative data from all sites relevant to the market is considered over an adequate period to even out normal fluctuations | Representative data from >50% of the sites relevant for the market considered over an adequate period to even out normal fluctuations | Representative data from only some sites (<<50%) relevant for the market considered OR >50% of sites, but from shorter periods | Representative data from only one site relevant for the market was considered OR some sites, but from shorter periods | Representativeness unknown or data from a small number of sites AND from shorter periods |
|  | 1.00 | 1.02 | 1.05 | 1.10 | 1.20 |
| 3 Temporal correlation | Less than 3 years of difference from our reference year | Less than 6 years of difference from our reference year | Less than 10 years of difference from our reference year | Less than 15 years of difference from our reference year | Age of data unknown or more than 15 years of difference to our reference year |
|  | 1.00 | 1.03 | 1.10 | 1.20 | 1.50 |
| 4 Geographical correlation | Data from the area under study | Average data from a larger area in which the area under study is included | Data from a smaller area than the area under study, or from a similar area | Data from an area with slightly similar production conditions | Data from unknown OR distinctly different area (North America instead of the Middle East, OECD-Europe instead of Russia) |
|  | 1.00 | 1.001 | 1.02 | 1.05 | 1.10 |
| 5 Further technological correlation | Data from enterprises, processes, and materials under study (i.e., identical technology) | Data from processes and materials under study (i.e., identical technology) but from different enterprises | Data on related processes or materials but the same technology, OR data from processes and materials under study but from different technology | Data on related processes or materials but different technology, OR data on laboratory-scale processes and the same technology | Data on related processes or materials, but on a laboratory scale of different technologies |
|  | 1.00 | 1.05 | 1.20 | 1.50 | 2.00 |

Table S4 Life cycle inventory

| Input/Output | Flow | Unit | Specific Cons. | Unit | Stream Data | Database | Data Quality |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dilution |  |  |  |  |  |  |  |
| Input |  |  |  |  |  |  |  |
| Vinasse | 2.64 | kg | 0.162462 | kg/kg TW | Vinasse wet, at processing {US} Mass, S | Agri-foodprint | 2,1,1,3,na |
| Water | 21.6 | kg | 1.329231 | kg/kg TW | Tap water {RoW}¦ market for tap water ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Electivity | 1.67 | Wh | 0.102769 | Wh/kg TW | Electricity, low voltage {ID}¦ market for electricity, low voltage ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,1,na |
| Output |  |  |  |  |  |  |  |
| Dilluted vinasse | 24.24 | kg | 1.491692 | kg/kg TW | Reference flow |  |  |
| Anaerobic |  |  |  |  |  |  |  |
| Input |  |  |  |  |  |  |  |
| Dilluted vinasse | 24.24 | kg | 1.491692 | kg/kg TW | Reference flow |  |  |
| Sludge | 0.66 | kg | 0.040615 | kg/kg TW | Digester sludge {RoW}¦ market for digester sludge ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Electricity | 2400 | Wh | 147.6923 | Wh/kg TW | Electricity, low voltage {ID}¦ market for electricity, low voltage ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,1,na |
| Output |  |  |  |  |  |  |  |
| Anaerobed water | 24.24 | kg | 1.491692 | kg/kg TW | Reference flow |  |  |
| Air emission |  |  |  |  |  |  |  |
| CH4 | 2.49 | g | 0.153231 | g/kg TW | Emission to air (unspecified) | SimaPro flow | 2,1,1,1,na |
| CO2 | 1.45 | g | 0.089231 | g/kg TW | Emission to air (unspecified) | SimaPro flow | 2,1,1,1,na |
| CO | 0.05 | g | 0.003077 | g/kg TW | Emission to air (unspecified) | SimaPro flow | 2,1,1,1,na |
| Aerobic |  |  |  |  |  |  |  |
| Input |  |  |  |  |  |  |  |
| Anaerobed water | 24.24 | kg | 1.491692 | kg/kg TW | Reference flow |  |  |
| Sludge | 0.253 | kg | 0.015569 | kg/kg TW | Digester sludge {RoW}¦ market for digester sludge ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Electricity | 8400 | Wh | 516.9231 | Wh/kg TW | Electricity, low voltage {ID}¦ market for electricity, low voltage ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,1,na |
| Output |  |  |  |  |  |  |  |
| Aerobed water | 24.24 | kg | 1.491692 | kg/kg TW | Reference flow |  |  |
| Air emission |  |  |  |  |  |  |  |
| CO2 | 8.08 | g | 0.497231 | g/kg TW | Emission to air (unspecified) | SimaPro flow | 2,1,1,1,na |
| Sedimentation |  |  |  |  |  |  |  |
| Input |  |  |  |  |  |  |  |
| Aerobed water | 24.24 | kg | 1.491692 | kg/kg TW | Reference flow |  |  |
| Electricity | 0.83 | Wh | 0.051077 | Wh/kg TW | Electricity, low voltage {ID}¦ market for electricity, low voltage ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,1,na |
| Output |  |  |  |  |  |  |  |
| Clarified water | 16.16 | kg | 0.994462 | kg/kg TW | Reference flow |  |  |
| Sludge | 8.08 | kg | 0.497231 | kg/kg TW | Reference flow |  |  |
| Electrochemical |  |  |  |  |  |  |  |
| Input |  |  |  |  |  |  |  |
| Clarified water | 16.16 | kg | 0.994462 | kg/kg TW | Reference flow |  |  |
| NaCl | 96.96 | g | 5.966769 | g/kg TW | Sodium chloride, powder {GLO}¦ market for sodium chloride, powder ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Ti/RuO2 (as TiO2) | 5.54E-02 | g | 0.003409 | g/kg TW | Titanium dioxide {RoW}¦ market for titanium dioxide ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,2,na |
| Ti/RuO2 (as Ru) | 5.54E-02 | g | 0.003409 | g/kg TW | Ruthenium {GLO} ¦ primary production, mining and processing ¦ production mix, at plant ¦ 12.37 g/cm3 ¦ LCI result | EF Database 3.1 | 2,1,1,2,na |
| Stainless stell | 1.94E-02 | g | 0.001194 | g/kg TW | Stainless steel quarto plate {RoW} ¦ rolling ¦ production mix, at plant ¦ stainless steel ¦ LCI result | EF Database 3.1 | 2,1,1,2,na |
| Electricity | 479.03 | Wh | 29.47877 | Wh/kg TW | Electricity, low voltage {ID}¦ market for electricity, low voltage ¦ Cut-off, S | Ecoinvent 3 | 2,1,1,1,na |
| Output |  |  |  |  |  |  |  |
| TW | 16.25 | kg | 1.00000 | kg/kg TW | Reference flow |  |  |
| Air emission |  |  |  |  |  |  |  |
| CO2 | 8.19 | g | 0.50400 | g/kg TW | Emission to air (unspecified) | SimaPro flow | 2,1,1,1,na |
| Cl2 | 11.9 | g | 0.732308 | g/kg TW | Emission to air (unspecified) | SimaPro flow | 2,1,1,1,na |

Table S5 LCIA of ReCiPe midpoint (H) 2016

| Impact category | Unit | Scenario 1 | Scenario 2 | Scenario 3 | Scenario 4 |
| --- | --- | --- | --- | --- | --- |
| Global warming | kg CO2 eq | 1.136761 | 0.855227 | 1.13164 | 0.85544 |
| Stratospheric ozone depletion | kg CFC11 eq | 1.10E-06 | 1.59E-06 | 1.10E-06 | 1.59E-06 |
| Ionizing radiation | kBq Co-60 eq | 0.00417 | 0.002684 | 0.00417 | 0.002696 |
| Ozone formation, Human health | kg NOx eq | 0.003155 | 0.002548 | 0.003155 | 0.002563 |
| Fine particulate matter formation | kg PM2.5 eq | 0.020763 | 0.015541 | 0.020763 | 0.015555 |
| Ozone formation, Terrestrial ecosystems | kg NOx eq | 0.003193 | 0.002472 | 0.003193 | 0.002487 |
| Terrestrial acidification | kg SO2 eq | 0.004545 | 0.003705 | 0.004545 | 0.003743 |
| Freshwater eutrophication | kg P eq | 0.001733 | 0.00129 | 0.001733 | 0.00129 |
| Marine eutrophication | kg N eq | 0.000209 | 0.000581 | 0.000209 | 0.000581 |
| Terrestrial ecotoxicity | kg 1,4-DCB | 2.207875 | 1.095639 | 2.20764 | 1.096256 |
| Freshwater ecotoxicity | kg 1,4-DCB | 0.073554 | 0.061871 | 0.073553 | 0.061871 |
| Marine ecotoxicity | kg 1,4-DCB | 0.088819 | 0.064026 | 0.088818 | 0.064026 |
| Human carcinogenic toxicity | kg 1,4-DCB | 0.106719 | 0.068274 | 0.106715 | 0.068273 |
| Human non-carcinogenic toxicity | kg 1,4-DCB | 1.814311 | 1.309457 | 1.814293 | 1.309512 |
| Land use | m2a crop eq | 0.081646 | 0.482715 | 0.081645 | 0.482992 |
| Mineral resource scarcity | kg Cu eq | 0.011327 | 0.009842 | 0.011327 | 0.010421 |
| Fossil resource scarcity | kg oil eq | 0.265047 | 0.190161 | 0.265043 | 0.191039 |
| Water consumption | m3 | 0.031667 | 0.02879 | 0.031667 | 0.029675 |

Table S6 Cumulative energy demand of vinasse wastewater treatment

| Impact category | Unit | Scenario 1 | Scenario 2 | Scenario 3 | Scenario 4 |
| --- | --- | --- | --- | --- | --- |
| Total | MJ | 12.88242 | 9.646986 | 12.88221 | 9.690107 |
| Non renewable, fossil | MJ | 12.00755 | 8.614541 | 12.00737 | 8.655014 |
| Non-renewable, nuclear | MJ | 0.105307 | 0.076346 | 0.105297 | 0.078389 |
| Non-renewable, biomass | MJ | 9.70E-05 | -3.06E-05 | 9.70E-05 | -3.06E-05 |
| Renewable, biomass | MJ | 0.020841 | 0.397049 | 0.020823 | 0.397035 |
| Renewable, wind, solar, geothermal | MJ | 0.492528 | 0.364324 | 0.492526 | 0.364571 |
| Renewable, water | MJ | 0.256099 | 0.194759 | 0.256095 | 0.195128 |

Table S7 Uncertainty analysis of GWP

| Scenario | Impact category | Unit | Mean | Median | SD | CoV (%) | 2.50% | 97.50% | SEM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario 1 | GWP100 - biogenic | kg CO2-eq | 0.038848 | 0.038834 | 0.001102 | 2.836433 | 0.036761 | 0.041073 | 1.10E-05 |
|  | GWP100 - fossil | kg CO2-eq | 1.084375 | 1.083934 | 0.023259 | 2.144891 | 1.039261 | 1.131311 | 0.000233 |
|  | GWP100 - land transformation | kg CO2-eq | 0.001716 | 0.001715 | 4.40E-05 | 2.563621 | 0.00163 | 0.001804 | 4.40E-07 |
| Scenario 2 | GWP100 - biogenic | kg CO2-eq | 0.030353 | 0.030333 | 0.000905 | 2.980806 | 0.028633 | 0.032175 | 9.05E-06 |
|  | GWP100 - fossil | kg CO2-eq | 0.814596 | 0.8144 | 0.017105 | 2.099813 | 0.781616 | 0.848582 | 0.000171 |
|  | GWP100 - land transformation | kg CO2-eq | 0.001231 | 0.00123 | 3.21E-05 | 2.609722 | 0.001169 | 0.001295 | 3.21E-07 |
| Scenario 3 | GWP100 - biogenic | kg CO2-eq | 0.034743 | 0.034718 | 0.001104 | 3.177487 | 0.03262 | 0.036937 | 1.10E-05 |
|  | GWP100 - fossil | kg CO2-eq | 1.083889 | 1.083459 | 0.023592 | 2.176627 | 1.039186 | 1.131633 | 0.000236 |
|  | GWP100 - land transformation | kg CO2-eq | 0.001715 | 0.001714 | 4.46E-05 | 2.603612 | 0.00163 | 0.001804 | 4.46E-07 |
| Scenario 4 | GWP100 - biogenic | kg CO2-eq | 0.027328 | 0.027317 | 0.000906 | 3.313957 | 0.025574 | 0.029136 | 9.06E-06 |
|  | GWP100 - fossil | kg CO2-eq | 0.813534 | 0.813461 | 0.017196 | 2.100786 | 0.785187 | 0.853122 | 0.000172 |
|  | GWP100 - land transformation | kg CO2-eq | 0.001231 | 0.001231 | 3.23E-05 | 2.626953 | 0.001169 | 0.001296 | 3.23E-07 |

SD: Standard deviation; CoV: Coefficient of variation; SEM: Standard error of mean