Comparative Simulation and Modeling of Hydrogen Explosion Consequences Based on TNT Equivalent

Supplementary Material

Supplementary Table S1. Classification of Structural Damage Risk Based on Peak Side-on Overpressure

| Risk damage (kPa) | Po (kPa) | Structural Damage Description | References |
| --- | --- | --- | --- |
| RD1 Insignificant (0.14 – ≤2) | ≤ 2 | Accidental glass damage. | (Wang et al., 2023) |
|  | 0.14 | Annoying noise (137 dB if low frequency, 10–15 Hz). | (Crowl and Louvar, 2011) |
|  | 0.21 | Breakage of large windows previously under stress. | (Crowl and Louvar, 2011) |
|  | 0.28 | Loud noise (143 dB), sonic boom, glass failure. | (Crowl and Louvar, 2011) |
|  | 0.69 | Small windows break due to pressure. | (Crowl and Louvar, 2011) |
|  | 1.03 | Typical pressure to break glass | (Crowl and Louvar, 2011) |
| RD2 Minor (>2 – ≤9) | > 2 − ≤9 | Minor glass breakage and light architectural damage such as displacement of roof tiles and falling plaster. | (Wang et al., 2023) |
|  | 2.07 | "Safe distance," 95% probability of no serious damage below this level, projectile limit, minor ceiling damage, 10% window breakage. | (Crowl and Louvar, 2011) |
|  | 2.76 | Limited minor structural damage. | (Crowl and Louvar, 2011) |
|  | 3.4 – 6.9 | Large and small windows usually shatter; occasional damage to window frames. | (Crowl and Louvar, 2011) |
|  | 4.8 | Minor structural damage to houses. | (Crowl and Louvar, 2011) |
|  | 6.9 | Partial house collapse, uninhabitable. | (Crowl and Louvar, 2011) |
|  | 9.0 | Slight distortion of coated steel frames. | (Crowl and Louvar, 2011) |
| RD3 Moderate (>9 – ≤25) | > 9 − ≤25 | Widespread glass shattering and significant damage to non-structural elements like doors, windows, and roofs. | (Wang et al., 2023) |
|  | 13.8 | Partial collapse of roof and walls. | (Crowl and Louvar, 2011) |
|  | 13.8 – 20.7 | Collapse of unreinforced concrete walls or cement blocks. | (Crowl and Louvar, 2011) |
|  | 15.8 | Lower limit of serious structural damage. | (Crowl and Louvar, 2011) |
|  | 17.2 | 50% of brick walls collapse | (Crowl and Louvar, 2011) |
|  | 20.7 | Heavy machinery (3,000 lb) slightly damaged in industrial buildings; steel frames distorted or detached from foundation. | (Crowl and Louvar, 2011) |
|  | 20.7 ─ ≤25 | Frame-less steel panels destroyed; oil storage tanks ruptured | (Crowl and Louvar, 2011) |
| RD4 Serious (>25 – ≤40) | > 25 - ≤40 | Extensive damage to non-structural elements such as brick facades, roofs, and ceilings. | (Wang et al., 2023) |
|  | 27.6 | Light industrial structures collapse. | (Crowl and Louvar, 2011) |
|  | 34.5 | Utility poles break; 40,000 lb hydraulic equipment slightly damaged. | (Crowl and Louvar, 2011) |
|  | 34.5 ─ ≤40 | Near-total destruction of residential houses. | (Crowl and Louvar, 2011) |
|  | 48.2 | Freight train cars completely destroyed. | (Crowl and Louvar, 2011) |
| RD5 Severe (>40 – ≤55) | > 40 − ≤55 | Heavy damage to non-structural elements leading to ceiling collapse. | (Wang et al., 2023) |
|  | 40 – 48.2 | Near-total destruction of residential houses. | (Crowl and Louvar, 2011) |
|  | 48.2 | Freight train cars completely destroyed. | (Crowl and Louvar, 2011) |
|  | 48.2 − ≤55 | 8–12-inch-thick unreinforced brick panels fail from shear or bending | (Crowl and Louvar, 2011) |
| RD6 Major (>55 – ≤76) | > 55 − 76 | Partial collapse of non-structural elements and significant damage to concrete columns | (Wang et al., 2023) |
|  | 55 – 55.1 | 8–12-inch-thick unreinforced brick panels fail from shear or bending. | (Crowl and Louvar, 2011) |
|  | 62 | Fully loaded train cars totally destroyed. | (Crowl and Louvar, 2011) |
|  | 68.9 | Likely total damage to structures; 7,000 lb equipment displaced and severely damaged; 12,000 lb machinery still present. | (Crowl and Louvar, 2011) |
| RD7 Catastrophic (>76) | > 76 | Near-total collapse of all building elements, including heavy damage to load-bearing concrete columns | (Wang et al., 2023) |
|  | 2068 | Limit of crater lip | (Crowl and Louvar, 2011) |

Supplementary Table S2. Classification of Human Injury Risk Based on Peak Side-on Overpressure

| Risk injury | Po (kPa) | Primary Effects | Secondary/Tertiary Effects | Conclusion |
| --- | --- | --- | --- | --- |
| RI1 Minor Injury | <20 | No serious injuries; eardrum rupture is not significant. | Glass cracking can begin around 1–1.5 kPa (Jeremić and Bajić, 2006).; widespread breakage typically 2–9 kPa (Wang et al., 2023). | Primary injury is unlikely, but secondary injury risk from glass fragments remains. |
| RI2 Moderate Injury | 20–30 | Minor contusion (Wang et al., 2023); 1% eardrum rupture (20–30 kPa) (Jeremić and Bajić, 2006). | Moderate structure damage may cause ceiling sections to fall and small objects to drop (Crowl and Louvar, 2011). | Combination of minor contusion and secondary injuries from fragments/debris. |
| RI3 Serious Injury | 30–50 | Moderate contusion (Wang et al., 2023); eardrum rupture probability 50% around 110 kPa (Jeremić and Bajić, 2006). | Wider structural damage creates flying debris (Crowl and Louvar, 2011), which can cause fractures. | Injury severity rises due to the combined effects of the blast wave and flying debris. |
| RI4 Severe Injury | 50–100 | Serious internal contusions and possibly fatal (Wang et al., 2023); risk of blast-lung depends on positive-phase duration with overpressure 70 kPa at 50 ms or 140–200 kPa at 3 ms (Jeremić and Bajić, 2006). | Many buildings heavily damaged; some may be totally destroyed (Crowl and Louvar, 2011), creating entrapment under rubble. | Concurrence of internal injuries and entrapment is potentially fatal. |
| RI5 Fatality | >100 | High mortality for unprotected persons, but thresholds depend on positive-phase duration: 1% lethality 190 kPa at 50 ms or 400–500 kPa at 3 ms (Jeremić and Bajić, 2006). | Total building collapse; large debris moving rapidly (Crowl and Louvar, 2011). | Fatality risk escalates where both overpressure and duration exceed thresholds. |

Supplementary Table S3. Hopkinson–Cranz scale-up consistency from the PVBs baseline to the Gangneung benchmark using adjusted distance (Radj)

| R (m) | Ze (m·kg⁻¹ᐟ³) | Radj (m) | Po (kPa) at WTNT 50 kg |  |  | Error (%) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Crowl | Alonso | Sadovski | Crowl | Alonso | Sadovski |
| 2 | 0.94 | 3.48 | 2282.466 | 1268.002 | 1376.581 | -0.22% | -0.20% | -0.27% |
| 4 | 1.89 | 6.95 | 476.954 | 315.719 | 248.702 | 0.10% | 0.08% | 0.10% |
| 6 | 2.83 | 10.43 | 187.438 | 139.618 | 102.779 | -0.01% | -0.01% | -0.01% |
| 8 | 3.77 | 13.91 | 100.912 | 78.272 | 58.455 | -0.06% | -0.06% | -0.06% |
| 10 | 4.72 | 17.39 | 65.148 | 50.025 | 39.14 | 0.02% | 0.03% | 0.03% |
| 14 | 6.60 | 24.34 | 36.376 | 25.421 | 22.524 | -0.03% | -0.04% | -0.03% |
| 18 | 8.49 | 31.30 | 24.964 | 15.344 | 15.48 | -0.01% | -0.01% | -0.01% |

Notes: (1) Positive error indicates overprediction; (2) negative error indicates underprediction