LAMPIRAN II

KODE PROGRAM

Kode Program Arduino

- Untuk Sensor Gas TGS 2442

/*****************************************************

#include <LiquidCrystal.h>

#include <SoftwareSerial.h>

LiquidCrystal lcd(13, 12, 11, 10, 9, 8);

SoftwareSerial Serial1(2, 3);

byte i;

float val1, CO, m2442, RS, RASIO, PPM;

int circ = 5, heat = 6, buzz = 7;

void setup() {

// put your setup code here, to run once:

lcd.begin(16, 2);

lcd.clear();

lcd.print("    TGS 2442    ");

lcd.setCursor(0, 1);

lcd.print("   CO Sensor    ");

Serial.begin(9600);

analogReference(EXTERNAL);

pinMode(circ, OUTPUT);

pinMode(heat, OUTPUT);

pinMode(buzz, OUTPUT);

analogWrite(heat, 255);

digitalWrite(circ, LOW);

Serial1.begin(9600);         // WiFly UART Baud Rate: 9600

delay(3000);

lcd.clear();

lcd.print("   Connecting   ");

lcd.setCursor(0, 1);

Serial1.print("$$$");

delay(500);

Serial1.print("set ip proto 0x2\r");

delay(500);

Serial1.print("set ip host 192.168.0.110\r");

delay(500);

Serial1.print("set ip remote 2000\r");

delay(500);

Serial1.print("open\r");

delay(500);

lcd.print("   Connected    ");

delay(3000);

}

void loop() {

// put your main code here, to run repeatedly:

GetCO();

lcd.clear();

lcd.print("   Sensor CO   ");

lcd.setCursor(0, 1);

lcd.print("  CO = ");

//  lcd.print(m2442);

if (CO <= 0)CO = 0;

Serial1.print('$');

Serial1.print(CO);

Serial1.print('#');

if (CO > 50) digitalWrite(buzz, HIGH);

else

digitalWrite(buzz, LOW);

lcd.print(CO);

lcd.print(" ppm ");

delay(1000);

}

void GetCO()

{

digitalWrite(circ, LOW);

analogWrite(heat, 245);

delay(14);

analogWrite(heat, 50);

delay(986);

digitalWrite(circ, HIGH);

delay(5);

val1 = 0;

for (int t = 1; t <= 100; t++) {

val1 += analogRead(A5);

}

digitalWrite(circ, LOW);

m2442 = val1 / 100;

//  Serial.print(m2442);

//  Serial.print('\n');

//  if (m2442 <=200) m2442=200;

//  if (m2442 >=270) m2442=270;

RS = map(m2442, 200, 270, 40916.5, 27878.78);

RASIO = RS / 12300;

CO = ((988 * RASIO) - 3999.64) / -3.92;

//  CO = map(val1, 100 , 200, 0, 100);

analogWrite(heat, 245);

}}/*****************************************************

- Untuk sensor Gas TGS 2620 dan 2602

/*****************************************************

#include <LiquidCrystal.h>

#include <SoftwareSerial.h>

LiquidCrystal lcd(13, 12, 11, 10, 9, 8);

SoftwareSerial Serial1(2, 3);

unsigned int  buzz = 7;

float ch, h2s, temp, m2620, m2602, RS, RASIO, PPM;

byte i;

void setup() {

// put your setup code here, to run once:

lcd.begin(16, 2);

lcd.clear();

lcd.print("    TGS 2602    ");

lcd.setCursor(0, 1);

lcd.print("    TGS 2620    ");

delay(90000);

Serial.begin(9600);

analogReference(EXTERNAL);

Serial1.begin(9600);         // WiFly UART Baud Rate: 9600

pinMode(buzz, OUTPUT);

delay(3000);

Serial1.print("$$$");

delay(500);

Serial1.print("set ip proto 0x2\r");

delay(500);

Serial1.print("set ip host 192.168.0.110\r");

delay(500);

Serial1.print("set ip remote 1000\r");

delay(500);

Serial1.print("open\r");

delay(3000);

}

void loop() {

// put your main code here, to run repeatedly:

temp = 0;

for (int x = 1; x <= 100; x++) {

temp += analogRead(A5);

}

m2620 = temp / 100;

if (m2620 <= 48) m2620 = 48;

if (m2620 >= 272) m2620 = 272;

RS = map(m2620, 48, 272, 20276.6, 2762.227);

RASIO = RS / 1000;

ch = ((4960 * RASIO) - 89892) / -15.3;

if (ch <= 0) ch = 0;

//ch = map(m2620, 50 , 250, 0, 100);

temp = 0;

for (int x = 1; x <= 100; x++) {

temp += analogRead(A4);

}

m2602 = temp / 100;

if (m2602 <= 5) m2602 = 5;

if (m2602 >= 200) m2602 = 200;

RS = map(m2602, 5, 200, 207333.3333, 4112.474438);

RASIO = RS / 10000;

h2s = ((1.9 * RASIO) - 1.575) / -0.55;

if (h2s <= 0) h2s = 0;

//h2s = map(m2602, 90 , 200, 0, 100);

lcd.clear();

lcd.setCursor(0, 0);

lcd.print("ch  = ");

lcd.print(ch);

lcd.print(" ppm ");

lcd.setCursor(0, 1);

lcd.print("h2s = ");

lcd.print(h2s);

lcd.print(" ppm ");

Serial1.print('$');

Serial1.print(ch);

Serial1.print(',');

Serial1.print(h2s);

Serial1.print('#');

if (ch > 300 || h2s > 10)digitalWrite(buzz, HIGH);

else

digitalWrite(buzz, LOW);

delay(1000);

}

- Program Delphi untuk  Tampilan Depan

Source Code :

unit Unit1;

interface

uses

Winapi.Windows, Winapi.Messages, System.SysUtils, System.Variants, System.Classes, Vcl.Graphics,

Vcl.Controls, Vcl.Forms, Vcl.Dialogs, Vcl.ExtCtrls, Vcl.StdCtrls,

Vcl.Imaging.jpeg;

type

TfrmCover = class(TForm)

Panel1: TPanel;

Label1: TLabel;

Panel2: TPanel;

Label2: TLabel;

Panel3: TPanel;

Panel4: TPanel;

Label3: TLabel;

Panel5: TPanel;

Panel6: TPanel;

Image1: TImage;

Panel7: TPanel;

Panel8: TPanel;

Label4: TLabel;

Panel9: TPanel;

Panel10: TPanel;

Panel11: TPanel;

Panel12: TPanel;

Label5: TLabel;

Label6: TLabel;

Label7: TLabel;

Label8: TLabel;

Label9: TLabel;

procedure Image1Click(Sender: TObject);

private

{ Private declarations }

public

{ Public declarations }

end;

var

frmCover: TfrmCover;

implementation

{$R *.dfm}

uses Unit2;

procedure TfrmCover.Image1Click(Sender: TObject);

begin

frmMain.ShowModal;

//mCover.Hide;

end;

end.

- Untuk Tampilan Logging Data

Source Code :

unit Unit2;

interface

uses

Winapi.Windows, Winapi.Messages, System.SysUtils, System.Variants, System.Classes, Vcl.Graphics,

Vcl.Controls, Vcl.Forms, Vcl.Dialogs, Data.DB, Vcl.StdCtrls, Vcl.ComCtrls,

Vcl.Grids, Vcl.DBGrids, ZAbstractRODataset, ZDataset,

ZAbstractConnection, ZConnection, ZSqlProcessor, VclTee.TeeGDIPlus,

VCLTee.Series, VCLTee.TeEngine, Vcl.ExtCtrls, VCLTee.TeeProcs, VCLTee.Chart,

VCLTee.DBChart, System.Win.ScktComp;

type

TfrmMain = class(TForm)

ZConnection1: TZConnection;

ZQsensor1: TZReadOnlyQuery;

ZQsensor2: TZReadOnlyQuery;

DataSource1: TDataSource;

DBGrid1: TDBGrid;

DataSource2: TDataSource;

DBGrid2: TDBGrid;

ZSQLP: TZSQLProcessor;

DBChart1: TDBChart;

Panel1: TPanel;

Panel2: TPanel;

DTP1: TDateTimePicker;

GroupBox1: TGroupBox;

Series1: TBarSeries;

ServerSocket1: TServerSocket;

ServerSocket2: TServerSocket;

BtnSens2: TButton;

Memo1: TMemo;

Label1: TLabel;

Label2: TLabel;

txtCh: TEdit;

txtH2s: TEdit;

Label3: TLabel;

Label4: TLabel;

DTP2: TDateTimePicker;

BtnFilter2: TButton;

btn2Save: TButton;

Label5: TLabel;

Label6: TLabel;

txtCO: TEdit;

BtnSens1: TButton;

btn1Save: TButton;

BtnFilter1: TButton;

DBChart2: TDBChart;

BarSeries1: TBarSeries;

Memo2: TMemo;

Series2: TBarSeries;

procedure BtnSens2Click(Sender: TObject);

procedure ServerSocket2ClientConnect(Sender: TObject;

Socket: TCustomWinSocket);

procedure ServerSocket2ClientRead(Sender: TObject;

Socket: TCustomWinSocket);

procedure BtnFilter2Click(Sender: TObject);

procedure btn2SaveClick(Sender: TObject);

procedure BtnSens1Click(Sender: TObject);

procedure btn1SaveClick(Sender: TObject);

procedure BtnFilter1Click(Sender: TObject);

procedure ServerSocket1ClientRead(Sender: TObject;

Socket: TCustomWinSocket);

private

{ Private declarations }

public

{ Public declarations }

end;

var

frmMain: TfrmMain;

ch,h2s,co:string;

d2save,d1save:Boolean;

implementation

{$R *.dfm}

procedure TfrmMain.BtnFilter1Click(Sender: TObject);

begin

ZQsensor1.Filtered:=false;

zqsensor1.Filter:='Date='+QuotedStr(FormatDateTime('yyyy-mm-dd',DTP1.Date));

ZQsensor1.Filtered:=true;

end;

procedure TfrmMain.BtnFilter2Click(Sender: TObject);

begin

ZQsensor2.Filtered:=false;

zqsensor2.Filter:='Date='+QuotedStr(FormatDateTime('yyyy-mm-dd',DTP2.Date));

ZQsensor2.Filtered:=true;

end;

procedure TfrmMain.btn1SaveClick(Sender: TObject);

begin

if d1save then

begin

btn1Save.Caption:='Save';

d1save:=False;

end

else

begin

btn1Save.Caption:='Stop Saving';

d1save:=True;;

end;

end;

procedure TfrmMain.btn2SaveClick(Sender: TObject);

begin

if d2save then

begin

btn2Save.Caption:='Save';

d2save:=False;

end

else

begin

btn2Save.Caption:='Stop Saving';

d2save:=True;;

end;

end;

procedure TfrmMain.BtnSens1Click(Sender: TObject);

begin

if ServerSocket1.Active = False then

begin

ServerSocket1.Active:=True;

BtnSens1.Caption:='Stop';

end

else

begin

ServerSocket1.Active:=False;

BtnSens1.Caption:='Start';

end;

end;

procedure TfrmMain.BtnSens2Click(Sender: TObject);

begin

if ServerSocket2.Active = False then

begin

ServerSocket2.Active:=True;

BtnSens2.Caption:='Stop';

end

else

begin

ServerSocket2.Active:=False;

BtnSens2.Caption:='Start';

end;

end;

//SELECT * FROM sensor01 s1,SENSOR02 s2 WHERE s1.date='' AND s1.date=s2.date

procedure TfrmMain.ServerSocket1ClientRead(Sender: TObject;

Socket: TCustomWinSocket);

var

s:string;

i,j:Integer;

stt:Boolean;

begin

Memo2.Clear;

Memo2.Text:=Socket.ReceiveText;

s:=Memo2.Text;

s:='';

stt:=False;

for I := 1 to length(Memo2.Text) do

begin

if copy(Memo2.Text,i,1)='$' then

begin

stt:=True;

j:=1;

co:='';

end

else

if copy(Memo2.Text,i,1)='#' then

begin

stt:=False;

j:=0;

txtCO.Text:=co;

if d1save then

begin

zsqlp.Script.Clear;

zsqlp.Script.Text:='Insert into sensor01 (date,time,co) VALUES('+QuotedStr(FormatDateTime('yyyy-mm-dd',Now))+','+

QuotedStr(FormatDateTime('HH:MM:SS',Now))+','+

QuotedStr(co)+')';

zsqlp.Execute;

ZQsensor1.Refresh;

end;

end

else

if stt then

co:=co+copy(Memo2.Text,i,1);

end;

end;

procedure TfrmMain.ServerSocket2ClientConnect(Sender: TObject;

Socket: TCustomWinSocket);

begin

Socket.SendText('Connected');

end;

procedure TfrmMain.ServerSocket2ClientRead(Sender: TObject;

Socket: TCustomWinSocket);

var

s:string;

i,j:Integer;

stt:Boolean;

begin

Memo1.Clear;

Memo1.Text:=Socket.ReceiveText;

s:=Memo1.Text;

s:='';

stt:=False;

for I := 1 to length(Memo1.Text) do

begin

if copy(Memo1.Text,i,1)='$' then

begin

stt:=True;

j:=1;

ch:='';

h2s:='';

end

else

if copy(Memo1.Text,i,1)='#' then

begin

stt:=False;

j:=0;

txtCh.Text:=ch;

txtH2s.Text:=h2s;

if d2save then

begin

zsqlp.Script.Clear;

zsqlp.Script.Text:='Insert into sensor02 (date,time,H2S,CH) VALUES('+QuotedStr(FormatDateTime('yyyy-mm-dd',Now))+','+

QuotedStr(FormatDateTime('HH:MM:SS',Now))+','+

QuotedStr(h2s)+','+QuotedStr(ch)+')';

zsqlp.Execute;

ZQsensor1.Refresh;

end;

end

else

if copy(Memo1.Text,i,1)=',' then

inc(j)

else

if stt then

begin

if j=1 then

ch:=ch+copy(Memo1.Text,i,1)

else

h2s:=h2s+copy(Memo1.Text,i,1);

end;

end;

end;

end.